Quick guide to pair and un pair NX Wind Tx with WSI box

To release the pairing on the Wind TX

- Press and hold the pairing button for min 5 sec until the led light goes down.

To release the paring on the WSI box:

- Turn on the power

- Enter the setup mode on the instrument (Press and hold the SET button)

NX Wind instrument:

- Change the C53 value from 1.70 to 1.99. Confirm with SET.

- Change the 1.99 value back to 1.70. Confirm with SET

NX Sea Data:

- Change the C15 value from 00.4 to 99.9. Confirm with SET

- Change the 99.9 value back to 00.4. Confirm with SET

NX2 Multi Control:

- Change the C22 value from 00.4 to 99.9. Confirm with SET

- Change the 99.9 value back to 00.4. Confirm with SET

To pair the Wind Tx with the WSI box:

- Push the pairing button on the wind Tx for 1 sec. The LED in the wind Tx will flash twice if the pairing was successful. If not try again.

It’s very important to have the wind Tx close to the WSI box when pairing the units.